/**************************************************************

FIRST RESPONDER CPS - PULSE SYNC

------------------------------------------------------------

Hardware:

ESP32 WROOM-32

MAX30102 Heart Rate / SpO2

MQ-2 Combustible Gas Sensor

MQ-7 CO Indicator

SSD1306 128x64 OLED

WiFi:

SSID     : Pulse Sync

Password : 12345678

Dashboard:

http://192.168.4.1

API:

http://192.168.4.1/api/data

IMPORTANT:

MQ-2 and MQ-7 values in this prototype are RELATIVE

changes from startup baseline. They are NOT calibrated

ppm or %LEL measurements.

**************************************************************/

#include <WiFi.h>

#include <WebServer.h>

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#include "MAX30105.h"

#include "spo2_algorithm.h"

// ============================================================

// PIN CONFIGURATION

// ============================================================

#define SDA_PIN 21

#define SCL_PIN 22

#define MQ2_PIN 34

#define MQ7_PIN 35

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

#define OLED_ADDRESS 0x3C

// ============================================================

// WIFI

// ============================================================

const char* ssid = "FIRST_RESPONDER";

const char* password = "12345678";

WebServer server(80);

// ============================================================

// OLED

// ============================================================

Adafruit_SSD1306 display(

SCREEN_WIDTH,

SCREEN_HEIGHT,

&Wire,

-1

);

bool oledOK = false;

// ============================================================

// MAX30102

// ============================================================

MAX30105 maxSensor;

bool maxOK = false;

uint32_t irBuffer[100];

uint32_t redBuffer[100];

int32_t spo2;

int8_t validSpO2;

int32_t heartRate;

int8_t validHeartRate;

float displayHR = 0;

float displaySpO2 = 0;

bool fingerDetected = false;

// ============================================================

// MQ SENSOR VARIABLES

// ============================================================

float mq2Baseline = 0;

float mq7Baseline = 0;

float mq2Filtered = 0;

float mq7Filtered = 0;

float mq2Raw = 0;

float mq7Raw = 0;

float mq2Change = 0;

float mq7Change = 0;

const float MQ_FILTER_ALPHA = 0.10;

// ============================================================

// SYSTEM STATUS

// ============================================================

String gasStatus = "WAIT";

String coStatus = "WAIT";

String systemStatus = "STARTING";

// ============================================================

// TIMING

// ============================================================

unsigned long lastGasRead = 0;

unsigned long lastOLED = 0;

unsigned long lastSerial = 0;

const unsigned long GAS_INTERVAL = 200;

const unsigned long OLED_INTERVAL = 500;

const unsigned long SERIAL_INTERVAL = 1000;

// ============================================================

// ADC AVERAGING

// ============================================================

float readADC(int pin)

{

long total = 0;

for (int i = 0; i < 20; i++)

{

total += analogRead(pin);

delayMicroseconds(500);

}

return total / 20.0;

}

// ============================================================

// PERCENT CHANGE

// ============================================================

float calculateChange(float value, float baseline)

{

if (baseline <= 0)

return 0;

float change =

((value - baseline) / baseline) * 100.0;

if (change < 0)

change = 0;

return change;

}

// ============================================================

// SENSOR STATUS

// ============================================================

String calculateStatus(float change)

{

if (change >= 50)

return "DANGER";

if (change >= 20)

return "WARNING";

return "SAFE";

}

// ============================================================

// UPDATE GAS SENSORS

// ============================================================

void updateGasSensors()

{

if (millis() - lastGasRead < GAS_INTERVAL)

return;

lastGasRead = millis();

mq2Raw = readADC(MQ2_PIN);

mq7Raw = readADC(MQ7_PIN);

mq2Filtered =

MQ_FILTER_ALPHA * mq2Raw +

(1.0 - MQ_FILTER_ALPHA) * mq2Filtered;

mq7Filtered =

MQ_FILTER_ALPHA * mq7Raw +

(1.0 - MQ_FILTER_ALPHA) * mq7Filtered;

mq2Change =

calculateChange(

mq2Filtered,

mq2Baseline

);

mq7Change =

calculateChange(

mq7Filtered,

mq7Baseline

);

gasStatus =

calculateStatus(mq2Change);

coStatus =

calculateStatus(mq7Change);

if (

gasStatus == "DANGER" ||

coStatus == "DANGER"

)

{

systemStatus = "DANGER";

}

else if (

gasStatus == "WARNING" ||

coStatus == "WARNING"

)

{

systemStatus = "WARNING";

}

else

{

systemStatus = "SAFE";

}

}

// ============================================================

// MAX30102 READING

// ============================================================

void updateVitals()

{

if (!maxOK)

return;

// Need 100 samples for Maxim algorithm

for (int i = 0; i < 100; i++)

{

unsigned long timeoutStart = millis();

while (!maxSensor.available())

{

maxSensor.check();

// Prevent permanent blocking

if (millis() - timeoutStart > 1000)

{

fingerDetected = false;

return;

}

server.handleClient();

}

redBuffer[i] =

maxSensor.getRed();

irBuffer[i] =

maxSensor.getIR();

maxSensor.nextSample();

// Keep web server responsive

if ((i % 10) == 0)

server.handleClient();

}

// ----------------------------------------------------------

// Calculate average IR

// ----------------------------------------------------------

uint64_t irTotal = 0;

for (int i = 0; i < 100; i++)

{

irTotal += irBuffer[i];

}

uint32_t irAverage =

irTotal / 100;

// ----------------------------------------------------------

// Finger detection

// ----------------------------------------------------------

if (irAverage < 10000)

{

fingerDetected = false;

displayHR = 0;

displaySpO2 = 0;

return;

}

fingerDetected = true;

// ----------------------------------------------------------

// Maxim HR + SpO2 Algorithm

// ----------------------------------------------------------

maxim_heart_rate_and_oxygen_saturation(

irBuffer,

100,

redBuffer,

&spo2,

&validSpO2,

&heartRate,

&validHeartRate

);

// ----------------------------------------------------------

// Heart rate validation

// ----------------------------------------------------------

if (

validHeartRate &&

heartRate >= 40 &&

heartRate <= 180

)

{

if (displayHR == 0)

{

displayHR = heartRate;

}

else

{

displayHR =

0.25 * heartRate +

0.75 * displayHR;

}

}

// ----------------------------------------------------------

// SpO2 validation

// ----------------------------------------------------------

if (

validSpO2 &&

spo2 >= 85 &&

spo2 <= 100

)

{

if (displaySpO2 == 0)

{

displaySpO2 = spo2;

}

else

{

displaySpO2 =

0.10 * spo2 +

0.90 * displaySpO2;

}

}

}

// ============================================================

// OLED DISPLAY

// ============================================================

void updateOLED()

{

if (!oledOK)

return;

if (millis() - lastOLED < OLED_INTERVAL)

return;

lastOLED = millis();

display.clearDisplay();

display.setTextColor(SSD1306_WHITE);

// ----------------------------------------------------------

// TITLE

// ----------------------------------------------------------

display.setTextSize(1);

display.setCursor(16, 0);

display.println("FIRST RESPONDER");

display.drawLine(

0,

10,

127,

10,

SSD1306_WHITE

);

// ----------------------------------------------------------

// HR

// ----------------------------------------------------------

display.setCursor(0, 16);

display.print("HR   : ");

if (

fingerDetected &&

displayHR > 0

)

{

display.print(

(int)displayHR

);

display.print(" BPM");

}

else

{

display.print("--");

}

// ----------------------------------------------------------

// SPO2

// ----------------------------------------------------------

display.setCursor(0, 28);

display.print("SpO2 : ");

if (

fingerDetected &&

displaySpO2 > 0

)

{

display.print(

(int)displaySpO2

);

display.print(" %");

}

else

{

display.print("--");

}

// ----------------------------------------------------------

// MQ-2

// ----------------------------------------------------------

display.setCursor(0, 41);

display.print("GAS  : ");

display.print(gasStatus);

// ----------------------------------------------------------

// MQ-7

// ----------------------------------------------------------

display.setCursor(0, 53);

display.print("CO   : ");

display.print(coStatus);

display.display();

}

// ============================================================

// SERIAL MONITOR

// ============================================================

void printSerial()

{

if (

millis() - lastSerial <

SERIAL_INTERVAL

)

return;

lastSerial = millis();

Serial.println();

Serial.println(

"================================"

);

Serial.print("HR          : ");

if (fingerDetected)

Serial.print(displayHR, 0);

else

Serial.print("--");

Serial.println(" BPM");

Serial.print("SpO2        : ");

if (fingerDetected)

Serial.print(displaySpO2, 0);

else

Serial.print("--");

Serial.println(" %");

Serial.print("MQ2 ADC     : ");

Serial.println(

mq2Filtered,

1

);

Serial.print("MQ2 Change  : ");

Serial.print(

mq2Change,

1

);

Serial.println("%");

Serial.print("Gas Status  : ");

Serial.println(

gasStatus

);

Serial.print("MQ7 ADC     : ");

Serial.println(

mq7Filtered,

1

);

Serial.print("MQ7 Change  : ");

Serial.print(

mq7Change,

1

);

Serial.println("%");

Serial.print("CO Status   : ");

Serial.println(

coStatus

);

Serial.print("SYSTEM      : ");

Serial.println(

systemStatus

);

Serial.print("Dashboard   : http://");

Serial.println(

WiFi.softAPIP()

);

}

// ============================================================

// JSON API

// ============================================================

void handleAPI()

{

String json;

json.reserve(350);

json = "{";

json += "\"hr\":";

json += String(

displayHR,

0

);

json += ",\"spo2\":";

json += String(

displaySpO2,

0

);

json += ",\"finger\":";

if (fingerDetected)

json += "true";

else

json += "false";

json += ",\"mq2\":";

json += String(

mq2Filtered,

1

);

json += ",\"mq2Change\":";

json += String(

mq2Change,

1

);

json += ",\"gasStatus\":\"";

json += gasStatus;

json += "\"";

json += ",\"mq7\":";

json += String(

mq7Filtered,

1

);

json += ",\"mq7Change\":";

json += String(

mq7Change,

1

);

json += ",\"coStatus\":\"";

json += coStatus;

json += "\"";

json += ",\"systemStatus\":\"";

json += systemStatus;

json += "\"";

json += ",\"max30102\":";

json += maxOK

? "true"

: "false";

json += ",\"oled\":";

json += oledOK

? "true"

: "false";

json += "}";

server.sendHeader(

"Cache-Control",

"no-cache"

);

server.send(

200,

"application/json",

json

);

}

// ============================================================

// PULSE SYNC WEB DASHBOARD

// ============================================================

const char INDEX_HTML[] PROGMEM = R"rawliteral(

<!DOCTYPE html>

<html lang="en">

<head>

<meta charset="UTF-8">

<meta name="viewport"

content="width=device-width,initial-scale=1.0">

<title>

Pulse Sync - First Responder CPS

</title>

<style>

:root

{

--page:#f1f6ee;

--card:#fffdf9;

--ink:#273a2b;

--dim:#728074;

--green:#2f6b4f;

--ok:#4caf6d;

--warn:#d98a34;

--danger:#c1543f;

--blue:#2f8f93;

--shadow:

0 8px 30px rgba(39,58,43,.08);

}

*

{

box-sizing:border-box;

}

body

{

margin:0;

font-family:

Arial,

Helvetica,

sans-serif;

background:var(--page);

color:var(--ink);

}

header

{

background:rgba(255,253,249,.95);

padding:18px 5%;

display:flex;

align-items:center;

justify-content:space-between;

border-bottom:

1px solid #dfe8db;

position:sticky;

top:0;

z-index:20;

}

.brand

{

display:flex;

align-items:center;

gap:12px;

}

.logo

{

width:46px;

height:46px;

border-radius:14px;

background:

linear-gradient(

135deg,

#4caf6d,

#2f6b4f

);

color:white;

display:flex;

align-items:center;

justify-content:center;

font-size:24px;

font-weight:bold;

}

.brand h1

{

margin:0;

font-size:22px;

}

.brand p

{

margin:3px 0 0;

font-size:12px;

color:var(--dim);

}

.connection

{

padding:9px 14px;

background:#e9f6ec;

border-radius:999px;

font-size:12px;

font-weight:bold;

color:#357b4d;

}

.connection.offline

{

background:#fbe9e6;

color:var(--danger);

}

.dot

{

display:inline-block;

width:8px;

height:8px;

border-radius:50%;

background:var(--ok);

margin-right:7px;

}

.shell

{

width:min(1200px,92%);

margin:28px auto 60px;

}

.hero

{

margin-bottom:20px;

}

.hero h2

{

margin:0;

font-size:25px;

}

.hero p

{

margin:6px 0 0;

color:var(--dim);

font-size:13px;

}

.topgrid

{

display:grid;

grid-template-columns:

1fr 1fr 1fr;

gap:16px;

}

.card

{

background:var(--card);

border-radius:20px;

padding:20px;

box-shadow:var(--shadow);

border:

1px solid rgba(47,107,79,.08);

}

.profile

{

min-height:150px;

}

.profiletop

{

display:flex;

align-items:center;

gap:14px;

}

.avatar

{

width:55px;

height:55px;

border-radius:17px;

background:#e7f2e6;

display:flex;

align-items:center;

justify-content:center;

font-size:20px;

font-weight:bold;

color:var(--green);

}

.profile h3

{

margin:0;

font-size:18px;

}

.small

{

color:var(--dim);

font-size:12px;

margin-top:5px;

}

.online

{

margin-top:22px;

display:inline-block;

padding:7px 11px;

background:#e9f6ec;

border-radius:10px;

color:#3f8154;

font-size:12px;

font-weight:bold;

}

.metric

{

min-height:150px;

position:relative;

overflow:hidden;

}

.metric.spo2

{

background:

linear-gradient(

135deg,

#e7f7f6,

#fffdf9

);

}

.metric.hr

{

background:

linear-gradient(

135deg,

#fff0e6,

#fffdf9

);

}

.label

{

color:var(--dim);

font-size:13px;

font-weight:bold;

}

.value

{

margin-top:22px;

font-size:45px;

font-weight:800;

letter-spacing:-2px;

}

.unit

{

font-size:13px;

color:var(--dim);

margin-top:4px;

}

.heart

{

position:absolute;

right:20px;

top:20px;

color:#d9773f;

font-size:26px;

animation:beat 1s infinite;

}

@keyframes beat

{

0%,100%

{

transform:scale(1);

}

50%

{

transform:scale(1.22);

}

}

.main

{

display:grid;

grid-template-columns:

minmax(0,1fr) 330px;

gap:16px;

margin-top:16px;

}

.title

{

font-size:16px;

font-weight:bold;

}

.subtitle

{

color:var(--dim);

font-size:12px;

margin-top:4px;

}

.chart

{

height:230px;

margin-top:20px;

background:#f7faf5;

border-radius:15px;

overflow:hidden;

position:relative;

}

canvas

{

width:100%;

height:100%;

}

.legend

{

display:flex;

gap:20px;

margin-top:12px;

color:var(--dim);

font-size:12px;

}

.legend span::before

{

content:"";

display:inline-block;

width:9px;

height:9px;

border-radius:50%;

margin-right:6px;

}

.legend .hrkey::before

{

background:#d9773f;

}

.legend .spkey::before

{

background:#2f8f93;

}

.hazard

{

margin-top:16px;

}

.sensorrow

{

margin-top:22px;

}

.sensorhead

{

display:flex;

justify-content:space-between;

font-size:13px;

margin-bottom:8px;

}

.bar

{

height:10px;

background:#edf1e9;

border-radius:20px;

overflow:hidden;

}

.fill

{

height:100%;

width:0%;

background:var(--ok);

border-radius:20px;

transition:

width .4s,

background .4s;

}

.statusbadge

{

padding:4px 8px;

border-radius:8px;

font-size:11px;

font-weight:bold;

background:#e9f6ec;

color:#3c8052;

}

.alert

{

padding:12px 0;

border-bottom:

1px solid #edf1e9;

font-size:12px;

}

.alert:last-child

{

border-bottom:none;

}

.alerttime

{

color:var(--dim);

font-size:10px;

margin-bottom:4px;

}

.systembox

{

margin-top:16px;

}

.systemrow

{

display:flex;

justify-content:space-between;

align-items:center;

padding:12px 0;

border-bottom:

1px solid #edf1e9;

font-size:13px;

}

.light

{

width:10px;

height:10px;

border-radius:50%;

background:var(--ok);

}

.bigstatus

{

margin-top:20px;

text-align:center;

padding:18px;

border-radius:15px;

background:#e9f6ec;

color:#357b4d;

font-size:22px;

font-weight:bold;

}

.footer

{

text-align:center;

color:var(--dim);

font-size:11px;

margin-top:25px;

}

@media(max-width:850px)

{

.topgrid

{

grid-template-columns:1fr;

}

.main

{

grid-template-columns:1fr;

}

header

{

align-items:flex-start;

gap:12px;

flex-direction:column;

}

}

</style>

</head>

<body>

<header>

<div class="brand">

<div class="logo">

♥️

</div>

<div>

<h1>

Pulse Sync

</h1>

<p>

Advanced First Responder CPS Monitoring

</p>

</div>

</div>

<div

class="connection"

id="connection">

<span class="dot"></span>

ESP32 CONNECTED

</div>

</header>

<div class="shell">

<div class="hero">

<h2>

Live Responder Monitor

</h2>

<p>

Real-time physiological and environmental telemetry

</p>

</div>

<div class="topgrid">

<div class="card profile">

<div class="profiletop">

<div class="avatar">

01

</div>

<div>

<h3>

Responder 01

</h3>

<div class="small">

Wrist-Worn CPS Node

</div>

</div>

</div>

<div

class="online"

id="unitStatus">

● TELEMETRY ACTIVE

</div>

</div>

<div class="card metric spo2">

<div class="label">

BLOOD OXYGEN

</div>

<div

class="value"

id="spo2">

--

</div>

<div class="unit">

SpO₂ %

</div>

</div>

<div class="card metric hr">

<div class="heart">

♥️

</div>

<div class="label">

HEART RATE

</div>

<div

class="value"

id="hr">

--

</div>

<div class="unit">

beats per minute

</div>

</div>

</div>

<div class="main">

<div>

<div class="card">

<div class="title">

Vitals Trend

</div>

<div class="subtitle">

Last 60 telemetry readings

</div>

<div class="chart">

<canvas

id="chart">

</canvas>

</div>

<div class="legend">

<span class="hrkey">

Heart Rate

</span>

<span class="spkey">

SpO₂

</span>

</div>

</div>

<div class="card hazard">

<div class="title">

Hazard & Gas Readings

</div>

<div class="subtitle">

Relative change from startup baseline

</div>

<div class="sensorrow">

<div class="sensorhead">

<span>

MQ-2 Combustible Gas

</span>

<span>

<b id="gasValue">

0.0%

</b>

&nbsp;

<span

class="statusbadge"

id="gasStatus">

SAFE

</span>

</span>

</div>

<div class="bar">

<div

class="fill"

id="gasBar">

</div>

</div>

</div>

<div class="sensorrow">

<div class="sensorhead">

<span>

MQ-7 CO Indicator

</span>

<span>

<b id="coValue">

0.0%

</b>

&nbsp;

<span

class="statusbadge"

id="coStatus">

SAFE

</span>

</span>

</div>

<div class="bar">

<div

class="fill"

id="coBar">

</div>

</div>

</div>

<div

class="small"

style="margin-top:20px">

Prototype indicators only.

MQ values are not calibrated ppm or %LEL.

</div>

</div>

</div>

<div>

<div class="card">

<div class="title">

Mission Alerts

</div>

<div class="subtitle">

Live event feed

</div>

<div

id="alerts">

<div class="alert">

<div class="alerttime">

SYSTEM

</div>

Pulse Sync dashboard started.

</div>

</div>

</div>

<div class="card systembox">

<div class="title">

System Status

</div>

<div class="subtitle">

Hardware connection

</div>

<div class="systemrow">

<span>

MAX30102

</span>

<span

class="light"

id="maxLight">

</span>

</div>

<div class="systemrow">

<span>

MQ-2

</span>

<span class="light">

</span>

</div>

<div class="systemrow">

<span>

MQ-7

</span>

<span class="light">

</span>

</div>

<div class="systemrow">

<span>

OLED

</span>

<span

class="light"

id="oledLight">

</span>

</div>

<div

class="bigstatus"

id="overall">

SAFE

</div>

</div>

</div>

</div>

<div class="footer">

Pulse Sync • First Responder Cyber-Physical Safety Prototype

</div>

</div>

<script>

// ============================================================

// DATA HISTORY

// ============================================================

let hrHistory = [];

let spo2History = [];

let previousSystemStatus = "";

let previousGasStatus = "";

let previousCOStatus = "";

// ============================================================

// ALERT

// ============================================================

function addAlert(message)

{

const box =

document.getElementById("alerts");

const alert =

document.createElement("div");

alert.className =

"alert";

const time =

new Date().toLocaleTimeString();

alert.innerHTML =

'<div class="alerttime">' +

time +

'</div>' +

message;

box.prepend(alert);

while (

box.children.length > 6

)

{

box.removeChild(

box.lastChild

);

}

}

// ============================================================

// STATUS COLOUR

// ============================================================

function statusColor(status)

{

if (status === "DANGER")

return "#c1543f";

if (status === "WARNING")

return "#d98a34";

return "#4caf6d";

}

// ============================================================

// STATUS BADGE

// ============================================================

function updateBadge(

id,

status

)

{

const element =

document.getElementById(id);

element.textContent =

status;

element.style.color =

statusColor(status);

if (status === "DANGER")

element.style.background =

"#f8e3df";

else if (status === "WARNING")

element.style.background =

"#fff0dc";

else

element.style.background =

"#e9f6ec";

}

// ============================================================

// DRAW GRAPH

// ============================================================

function drawChart()

{

const canvas =

document.getElementById("chart");

const rect =

canvas.getBoundingClientRect();

canvas.width =

rect.width *

window.devicePixelRatio;

canvas.height =

rect.height *

window.devicePixelRatio;

const ctx =

canvas.getContext("2d");

ctx.scale(

window.devicePixelRatio,

window.devicePixelRatio

);

const W =

rect.width;

const H =

rect.height;

ctx.clearRect(

0,

0,

W,

H

);

// Grid

ctx.strokeStyle =

"#e4ebe0";

ctx.lineWidth =

1;

for (

let y = 30;

y < H;

y += 40

)

{

ctx.beginPath();

ctx.moveTo(

0,

y

);

ctx.lineTo(

W,

y

);

ctx.stroke();

}

// ----------------------------------------------------------

// HR GRAPH

// ----------------------------------------------------------

if (

hrHistory.length > 1

)

{

ctx.beginPath();

ctx.strokeStyle =

"#d9773f";

ctx.lineWidth =

2.5;

hrHistory.forEach(

(value, index) =>

{

const x =

index /

(hrHistory.length - 1) *

W;

const limited =

Math.max(

40,

Math.min(

180,

value

)

);

const y =

H -

((limited - 40) /

140) *

H;

if (index === 0)

ctx.moveTo(x, y);

else

ctx.lineTo(x, y);

}

);

ctx.stroke();

}

// ----------------------------------------------------------

// SPO2 GRAPH

// ----------------------------------------------------------

if (

spo2History.length > 1

)

{

ctx.beginPath();

ctx.strokeStyle =

"#2f8f93";

ctx.lineWidth =

2.5;

spo2History.forEach(

(value, index) =>

{

const x =

index /

(spo2History.length - 1) *

W;

const limited =

Math.max(

85,

Math.min(

100,

value

)

);

const y =

H -

((limited - 85) /

15) *

H;

if (index === 0)

ctx.moveTo(x, y);

else

ctx.lineTo(x, y);

}

);

ctx.stroke();

}

}

// ============================================================

// GET DATA FROM ESP32

// ============================================================

async function updateDashboard()

{

try

{

const response =

await fetch(

"/api/data",

{

cache:"no-store"

}

);

if (!response.ok)

throw new Error(

"API error"

);

const data =

await response.json();

// --------------------------------------------------------

// CONNECTION

// --------------------------------------------------------

const connection =

document.getElementById(

"connection"

);

connection.classList.remove(

"offline"

);

connection.innerHTML =

'<span class="dot"></span>' +

'ESP32 CONNECTED';

// --------------------------------------------------------

// HR / SPO2

// --------------------------------------------------------

if (

data.finger &&

data.hr > 0

)

{

document.getElementById(

"hr"

).textContent =

Math.round(data.hr);

hrHistory.push(

data.hr

);

if (

hrHistory.length > 60

)

hrHistory.shift();

}

else

{

document.getElementById(

"hr"

).textContent =

"--";

}

if (

data.finger &&

data.spo2 > 0

)

{

document.getElementById(

"spo2"

).textContent =

Math.round(data.spo2);

spo2History.push(

data.spo2

);

if (

spo2History.length > 60

)

spo2History.shift();

}

else

{

document.getElementById(

"spo2"

).textContent =

"--";

}

drawChart();

// --------------------------------------------------------

// MQ-2

// --------------------------------------------------------

document.getElementById(

"gasValue"

).textContent =

data.mq2Change.toFixed(1) +

"%";

document.getElementById(

"gasBar"

).style.width =

Math.min(

data.mq2Change,

100

) + "%";

document.getElementById(

"gasBar"

).style.background =

statusColor(

data.gasStatus

);

updateBadge(

"gasStatus",

data.gasStatus

);

// --------------------------------------------------------

// MQ-7

// --------------------------------------------------------

document.getElementById(

"coValue"

).textContent =

data.mq7Change.toFixed(1) +

"%";

document.getElementById(

"coBar"

).style.width =

Math.min(

data.mq7Change,

100

) + "%";

document.getElementById(

"coBar"

).style.background =

statusColor(

data.coStatus

);

updateBadge(

"coStatus",

data.coStatus

);

// --------------------------------------------------------

// OVERALL STATUS

// --------------------------------------------------------

const overall =

document.getElementById(

"overall"

);

overall.textContent =

data.systemStatus;

overall.style.color =

statusColor(

data.systemStatus

);

if (

data.systemStatus ===

"DANGER"

)

overall.style.background =

"#f8e3df";

else if (

data.systemStatus ===

"WARNING"

)

overall.style.background =

"#fff0dc";

else

overall.style.background =

"#e9f6ec";

// --------------------------------------------------------

// HARDWARE STATUS

// --------------------------------------------------------

document.getElementById(

"maxLight"

).style.background =

data.max30102

? "#4caf6d"

: "#c1543f";

document.getElementById(

"oledLight"

).style.background =

data.oled

? "#4caf6d"

: "#c1543f";

// --------------------------------------------------------

// ALERT CHANGES

// --------------------------------------------------------

if (

previousGasStatus !== "" &&

previousGasStatus !==

data.gasStatus

)

{

addAlert(

"MQ-2 gas status changed to <b>" +

data.gasStatus +

"</b>"

);

}

if (

previousCOStatus !== "" &&

previousCOStatus !==

data.coStatus

)

{

addAlert(

"MQ-7 CO indicator changed to <b>" +

data.coStatus +

"</b>"

);

}

if (

previousSystemStatus !== "" &&

previousSystemStatus !==

data.systemStatus

)

{

addAlert(

"System status changed to <b>" +

data.systemStatus +

"</b>"

);

}

previousGasStatus =

data.gasStatus;

previousCOStatus =

data.coStatus;

previousSystemStatus =

data.systemStatus;

}

catch(error)

{

const connection =

document.getElementById(

"connection"

);

connection.classList.add(

"offline"

);

connection.innerHTML =

"● ESP32 OFFLINE";

}

}

// ============================================================

// START

// ============================================================

updateDashboard();

setInterval(

updateDashboard,

1000

);

window.addEventListener(

"resize",

drawChart

);

</script>

</body>

</html>

)rawliteral";

// ============================================================

// ROOT WEB PAGE

// ============================================================

void handleRoot()

{

server.send_P(

200,

"text/html",

INDEX_HTML

);

}

// ============================================================

// 404

// ============================================================

void handleNotFound()

{

server.send(

404,

"text/plain",

"Not Found"

);

}

// ============================================================

// MQ BASELINE CALIBRATION

// ============================================================

void calibrateMQSensors()

{

Serial.println();

Serial.println(

"MQ SENSOR WARMUP"

);

Serial.println(

"Keep sensors in normal clean air."

);

// Short prototype warm-up

// Real MQ sensor conditioning is much longer.

for (int i = 30; i > 0; i--)

{

Serial.print(

"Warmup: "

);

Serial.print(i);

Serial.println(

" seconds"

);

if (oledOK)

{

display.clearDisplay();

display.setTextColor(

SSD1306_WHITE

);

display.setTextSize(1);

display.setCursor(

10,

15

);

display.println(

"MQ SENSOR WARMUP"

);

display.setCursor(

32,

35

);

display.print(i);

display.println(

" sec"

);

display.display();

}

delay(1000);

}

Serial.println();

Serial.println(

"CALIBRATING BASELINE..."

);

float total2 = 0;

float total7 = 0;

for (int i = 0; i < 30; i++)

{

total2 +=

readADC(MQ2_PIN);

total7 +=

readADC(MQ7_PIN);

delay(100);

}

mq2Baseline =

total2 / 30.0;

mq7Baseline =

total7 / 30.0;

mq2Filtered =

mq2Baseline;

mq7Filtered =

mq7Baseline;

Serial.print(

"MQ2 BASELINE = "

);

Serial.println(

mq2Baseline,

1

);

Serial.print(

"MQ7 BASELINE = "

);

Serial.println(

mq7Baseline,

1

);

}

// ============================================================

// SETUP

// ============================================================

void setup()

{

Serial.begin(115200);

delay(1000);

Serial.println();

Serial.println(

"================================"

);

Serial.println(

"FIRST RESPONDER CPS"

);

Serial.println(

"PULSE SYNC"

);

Serial.println(

"================================"

);

// ----------------------------------------------------------

// I2C

// ----------------------------------------------------------

Wire.begin(

SDA_PIN,

SCL_PIN

);

// ----------------------------------------------------------

// OLED

// ----------------------------------------------------------

oledOK =

display.begin(

SSD1306_SWITCHCAPVCC,

OLED_ADDRESS

);

if (oledOK)

{

Serial.println(

"[OK] OLED 0x3C"

);

display.clearDisplay();

display.setTextColor(

SSD1306_WHITE

);

display.setTextSize(1);

display.setCursor(

20,

15

);

display.println(

"PULSE SYNC"

);

display.setCursor(

8,

35

);

display.println(

"SYSTEM STARTING..."

);

display.display();

}

else

{

Serial.println(

"[ERROR] OLED"

);

}

// ----------------------------------------------------------

// MAX30102

// ----------------------------------------------------------

maxOK =

maxSensor.begin(

Wire,

I2C_SPEED_FAST

);

if (maxOK)

{

Serial.println(

"[OK] MAX30102"

);

maxSensor.setup(

50,

4,

2,

100,

411,

4096

);

maxSensor.setPulseAmplitudeRed(

0x1F

);

maxSensor.setPulseAmplitudeIR(

0x1F

);

maxSensor.setPulseAmplitudeGreen(

0

);

}

else

{

Serial.println(

"[ERROR] MAX30102"

);

}

// ----------------------------------------------------------

// ADC

// ----------------------------------------------------------

analogReadResolution(12);

analogSetPinAttenuation(

MQ2_PIN,

ADC_11db

);

analogSetPinAttenuation(

MQ7_PIN,

ADC_11db

);

// ----------------------------------------------------------

// MQ CALIBRATION

// ----------------------------------------------------------

calibrateMQSensors();

systemStatus =

"SAFE";

// ----------------------------------------------------------

// WIFI ACCESS POINT

// ----------------------------------------------------------

Serial.println();

Serial.println(

"STARTING WIFI..."

);

WiFi.mode(

WIFI_AP

);

bool apStarted =

WiFi.softAP(

ssid,

password

);

if (apStarted)

{

Serial.println(

"[OK] WIFI AP"

);

}

else

{

Serial.println(

"[ERROR] WIFI AP"

);

}

Serial.print(

"SSID      : "

);

Serial.println(

ssid

);

Serial.print(

"PASSWORD  : "

);

Serial.println(

password

);

Serial.print(

"IP        : "

);

Serial.println(

WiFi.softAPIP()

);

// ----------------------------------------------------------

// WEB ROUTES

// ----------------------------------------------------------

server.on(

"/",

HTTP_GET,

handleRoot

);

server.on(

"/api/data",

HTTP_GET,

handleAPI

);

server.onNotFound(

handleNotFound

);

server.begin();

Serial.println(

"[OK] WEB SERVER"

);

Serial.println();

Serial.println(

"OPEN:"

);

Serial.print(

"http://"

);

Serial.println(

WiFi.softAPIP()

);

Serial.println();

Serial.println(

"SYSTEM READY"

);

}

// ============================================================

// MAIN LOOP

// ============================================================

void loop()

{

// Web requests

server.handleClient();

// Gas sensors

updateGasSensors();

// OLED

updateOLED();

// Serial monitor

printSerial();

// MAX30102

//

// This obtains a complete 100-sample

// measurement window.

updateVitals();

// Handle any waiting browser request

server.handleClient();

}